Date:	July 29, 2026						Bid Stage:

QUALIFICATIONS:

PROJECT:

Net Sqft:

Scale:

Addendum and RFI

Q:

A:

Take-off based on plans and specs dated

Carpet:

0001, Mohawk Group/Lees Carpets/Custom, K1109 Dark Grey, High Road, 24" X 24", Monolithic Pattern

Not verified

0005, Matting By Design, Entry Vestibule Flooring: Ultra Dry, Ribbed Pattern

Size not found, 24”x24” taken for estimation

https://mattingbydesign.com/heavy-duty-logo-floor-mats

0009, Smartcells Mat, Anti-Fatigue Mat, Black, 24" X 48"

https://762e0728.flowpaper.com/SmartCellsProductLookbookv10AFpages/#page=5

Location: Cashwrap

Resilient: (LVT/VCT/Plank/Sheet)

RF-1, Karndean Design, Commercial Luxury Vinyl Tile, T-201, 19.7" x 24", Ashlar Pattern Running Length Of Room

https://www.karndean.com/en/floors/products/colorado-llt201

Location: Break Room And Asset Protection

Sqft/carton: 32.83

Size Is Taken From MFR

Floor Tile:

CT-1 (Floor), Daltile, Avery Glazed Porcelain, Grey AV19, 11.8125" X 23.8125”, ⅓ Offset Ashlar Pattern

https://digitalassets.daltile.com/content/dam/Daltile/website/resources/products/fact-sheets/avery/DAL_Avery_FS.pdf

Location: Men's Restroom / Women´s Restroom floor tile

Tile TH: 0.3125”

Sqft/carton: 15.60

Grout: G-1, Mapei, #11, Sahara Beige 1/8”

Wall Tile:

CT-3 (Wall), Daltile, Modern Dimensions: Ceramic, Matte Arctic White 0790, 4.25" X 12.875", ⅓ Offset Running Bond

https://digitalassets.daltile.com/content/dam/Daltile/website/resources/products/sales-sheets/modern-dimensions/DAL_ModernDimensions_SS.pdf

Location: Men's Restroom / Women´s Restroom wall tile

Tile TH: 0.3125”

Sft/carton: 10.64

Grout: G-2, Mapei, #38, Avalanche, 1/8”

https://digitalassets.daltile.com/content/dam/Daltile/website/resources/products/sales-sheets/modern-dimensions/DAL_ModernDimensions_SS.pdf

Bases:

CT-2 (Tile Base), Daltile, Portfolio Color Body Porcelain, Dove Grey PF04, 6" X 24”

https://digitalassets.daltile.com/content/dam/Daltile/website/resources/products/sales-sheets/portfolio/DAL_Portfolio_SS.pdf

Location: Men's Restroom / Women´s Restroom wall tile base

Pieces/carton: 22

Grout: G-1, Mapei, #11, Sahara Beige 1/8”

RB-1 (Base), Johnsonite, 29 Moon Rock, 6" High Cove Base

Location: Sales Floor: Direct-Glue Carpet, Polished Concrete, And Resilient Flooring; Dressing Rooms: Entry, Lounge And Outside Dressing Room Area

RB-2 (Base), Johnsonite, 29 Moon Rock, 4" High Cove Base

Location: Overall: Back Of House And Casework; Dressing Rooms: Inside Dressing Room Partitions

Concrete:

0003, Sealed Concrete

0004, Polished Concrete

0008, Ameripolish Concrete Dye, Stained Concrete, Midnight Black

Location: Sales Floor Aisles

Accessories:(Transitions/Heatweld/ Threshold)

A803/ C11 (Ceramic Tile To Concrete), Pemko Sbr.5, Rubber Ramp Style Threshold/ Metal Screed

A803/ C9 (Resilient Flooring To Concrete), Resilient Reducer Strip

A803/ C7 (Carpet To Resilient), Resilient Reducer Strip

A803/ C5 (Glue-Down Carpet To Concrete), Resilient Reducer Strip

A803/ I14(Threshold At Interior Door), Metal Threshold - Bed In Sealant

A803/ I6(Ceramic Tile Flooring To Resilient Flooring), Resilient Reducer Strip

Bid Alternate

Trims:

Takeoff Notes: